FRAME REINFORCEMENT MANUAL

Step One: Cut all green lines as seen in pictures below, Notch spring hangers

Step Two: Remove spring hangers and grind all welds down.

Step Three: Remove Park brake cable hanger from driver side

Step Four: Start at the bumper and measure 8 inches back on the frame, that will be where you place the reinforcement plate.

Step Five: Weld on the reinforcement plates to the frame of the truck.

Step Six: Weld the spring hangers to the reinforcement plates you welded onto the frame.

***Tip: When welding the frame reinforcement onto the frame follow the outside edge***

***Tip: Use a heat shield up against the fuel tank when welding near***